COMPUTER VISION WITH PYTHON & OPENCV

DAY 04 — REAL-TIME VIDEO PROCESSING

Reading, Processing, Analyzing, and Displaying Every Video Frame

| Duration | 6 hours |
| --- | --- |
| Programming | Python + OpenCV |
| Main Topic | Video processing and real-time computer vision |
| Main Skills | VideoCapture, frame processing, FPS, display, contours, motion detection |
| Main Project | Real-time traffic/motion analysis system |

# 1. Learning Objectives

By the end of Day 04, students should be able to:

Read video files frame by frame using cv2.VideoCapture().

Understand the relationship between a video and a sequence of images.

Apply OpenCV image-processing operations to every video frame.

Display original and processed frames in real time.

Measure processing speed using FPS and frame-processing time.

Use thresholding, edge detection, morphology, and contours on video.

Detect motion using frame differencing or background subtraction.

Draw bounding boxes and basic measurements around detected objects.

Save processed video using cv2.VideoWriter().

Design a complete, reproducible real-time computer-vision pipeline.

# 2. Day 04 Schedule

| Time | Session | Topics | Hands-on |
| --- | --- | --- | --- |
| 09:00–10:00 | Session 1 | Video fundamentals; frames, FPS, resolution, codecs | Inspect video properties |
| 10:00–11:00 | Session 2 | cv2.VideoCapture; frame-by-frame loop; keyboard control | Build a video reader |
| 11:00–12:00 | Session 3 | Frame processing: grayscale, blur, Canny, thresholding | Process every frame |
| 12:00–12:30 | Break | — | — |
| 12:30–13:30 | Session 4 | Contours, morphology, connected components | Detect regions/objects |
| 13:30–14:30 | Session 5 | Motion detection and FPS measurement | Real-time motion detector |
| 14:30–15:00 | Session 6 | VideoWriter, testing, assignment briefing | Save and evaluate results |

# 3. Session 1 — Understanding Video as a Sequence of Frames

A digital video can be treated as a sequence of images displayed at a specified frame rate. Students should understand that the same image-processing operations learned previously can be applied repeatedly to every frame.

Frame: one image from the video.

FPS (Frames Per Second): number of frames displayed/processed per second.

Resolution: width × height of each frame.

Duration: total playing time of the video.

Codec/container: how the video is encoded and stored.

Exercise 1 — Inspect a Video

Load a video with cv2.VideoCapture().

Print width, height, FPS, frame count, and duration.

Display the first frame.

Confirm that the video opens successfully.

Release the capture object and close the window.

# 4. Session 2 — Reading and Displaying Video in Real Time

Students will implement the basic processing loop:

Video → Read Frame → Process Frame → Display → Repeat

Core OpenCV functions:

cv2.VideoCapture()

cap.isOpened()

cap.read()

cv2.imshow()

cv2.waitKey()

cap.release()

cv2.destroyAllWindows()

Exercise 2 — Real-Time Video Reader

Read the video continuously.

Display every frame.

Stop with the ESC key or the 'q' key.

Display the current frame number.

# 5. Session 3 — Processing Every Frame

Apply the image-processing techniques from Days 1–3 to a live sequence of frames.

Grayscale conversion

Gaussian or median filtering

Canny edge detection

Sobel/Scharr edge detection

Global/Otsu/adaptive thresholding

Erosion, dilation, opening, and closing

Exercise 3 — Three Processing Pipelines

| Pipeline | Operations | Purpose |
| --- | --- | --- |
| A | Grayscale | Observe basic frame transformation |
| B | Grayscale → Gaussian Blur → Canny | Extract edges |
| C | Grayscale → Blur → Canny → Morphology | Clean and improve edge result |

# 6. Session 4 — Contours and Object Regions

Students will convert processed frames into useful object regions and investigate how contours can be used for localization.

cv2.findContours()

cv2.contourArea()

cv2.boundingRect()

cv2.drawContours()

Filtering contours by area

Bounding boxes

Connected components

Exercise 4 — Object Region Detection

Create a binary or edge image.

Find contours.

Remove very small contours.

Draw bounding rectangles around remaining regions.

Display the number of detected regions.

# 7. Session 5 — Motion Detection

The main practical application is to detect objects that change position between consecutive frames.

Frame differencing

Background subtraction

cv2.createBackgroundSubtractorMOG2()

cv2.createBackgroundSubtractorKNN()

Morphological cleanup

Contour-based localization

Recommended motion-detection pipeline:

Frame → Background Subtraction → Threshold → Morphology → Contours → Bounding Boxes

# 8. FPS and Real-Time Performance

Students must distinguish between the FPS stored in the source video and the actual FPS achieved by their processing program.

Measure elapsed processing time.

Calculate actual processing FPS.

Display FPS on the video frame.

Compare simple and complex processing pipelines.

Explain why computationally expensive operations reduce FPS.

Suggested measurements:

| Pipeline | Avg. Time/Frame (ms) | FPS | Visual Quality |
| --- | --- | --- | --- |

# 9. Saving the Processed Video

Students will learn to save the processed output using cv2.VideoWriter(). The output should preserve the original frame rate and appropriate frame size.

Select a video codec.

Set output FPS.

Set output frame size.

Write every processed frame.

Release the VideoWriter correctly.

# 10. Main Practical Assignment

Develop a real-time video-processing application using a real-world traffic or pedestrian video.

Minimum requirements:

Open a real video using cv2.VideoCapture().

Read every frame.

Apply at least three image-processing operations.

Display the original and processed results.

Display frame number and FPS.

Detect regions/objects using contours or motion detection.

Draw bounding boxes around detected regions.

Measure processing performance.

Save a processed output video.

Test the program on a second video without changing the core algorithm.

# 11. Recommended Dataset

Use a small number of real videos from Kaggle for development and testing. The Road Traffic Video Monitoring dataset is suitable because it contains several traffic videos.

Kaggle dataset: Road Traffic Video Monitoring — search Kaggle for: shawon10/road-traffic-video-monitoring

Recommended workflow:

Download one video for development.

Develop and debug the algorithm.

Download a second video for testing.

Run the same algorithm without retuning it for the second video.

Compare FPS and detection behavior.

# 12. Required Project Structure

Day_04_Real_Time_Video/
│
├── data/
│   ├── input_video_01.mp4
│   └── input_video_02.mp4
│
├── src/
│   └── real_time_video.py
│
├── results/
│   ├── screenshots/
│   └── processed_video.mp4
│
├── report/
│
├── README.md
└── requirements.txt

# 13. Deliverables

Python source code.

requirements.txt.

README.md explaining how to run the project.

Source video information/link.

20–30 second screen recording showing the application running.

At least four screenshots showing intermediate and final results.

A short technical report.

Performance table containing FPS and processing time.

Comparison between two test videos.

# 14. Technical Report Structure

Introduction

Video Dataset and Source

Problem Definition

Processing Pipeline

OpenCV Functions Used

Motion/Object Detection Method

Experimental Results

FPS and Processing-Time Analysis

Comparison of Test Videos

Failure Cases

Possible Improvements

Conclusion

# 15. Advanced / Bonus Tasks

Implement both frame differencing and MOG2 and compare them.

Track detected objects across frames.

Count the number of moving objects.

Estimate object speed in pixels/second.

Add a region of interest (ROI).

Add a virtual counting line and count objects crossing it.

Save detection statistics to CSV.

Create a side-by-side original/processed output.

Optimize the pipeline to increase FPS.

# 16. Assessment Rubric

| Component | Weight | Criteria |
| --- | --- | --- |
| Video handling | 15% | Correct frame-by-frame capture and display |
| Image processing | 20% | Correct and meaningful OpenCV processing |
| Detection/segmentation | 20% | Contours, motion, or object-region analysis |
| Real-time performance | 15% | FPS measurement and performance analysis |
| Experiments | 10% | Comparison of multiple processing pipelines |
| Code organization | 10% | Clear, modular, reproducible project |
| Report & presentation | 10% | Technical explanation, results, and discussion |

# 17. Key Learning Outcome

The goal is to move from processing ONE image to building a system that processes an ENTIRE VIDEO in real time.

Video → Frame → Process → Analyze → Display → Measure → Next Frame